|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.107 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (05/2000) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS International telephone connections and circuits – General definitions |  |  |  |
|  | The E-model, a computational model for use in transmission planning |  |  |  |
|  | ITU-T Recommendation G.107 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| General definitions | G.100–G.109 |
| General Recommendations on the transmission quality for an entire international telephone connection | G.110–G.119 |
| General characteristics of national systems forming part of international connections | G.120–G.129 |
| General characteristics of the 4-wire chain formed by the international circuits and national extension circuits | G.130–G.139 |
| General characteristics of the 4-wire chain of international circuits; international transit | G.140–G.149 |
| General characteristics of international telephone circuits and national extension circuits | G.150–G.159 |
| Apparatus associated with long-distance telephone circuits | G.160–G.169 |
| Transmission plan aspects of special circuits and connections using the international telephone connection network | G.170–G.179 |
| Protection and restoration of transmission systems | G.180–G.189 |
| Software tools for transmission systems | G.190–G.199 |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.107 The E-model, a computational model for use in transmission planning

- Summary This ITU-T Recommendation gives the algorithm for the so-called E-model as the common ITU-T Transmission Rating Model. This computational model can be useful to transmission planners, to help ensure that users will be satisfied with end-to-end transmission performance. The primary output of the model is a scalar rating of transmission quality. A major feature of this model is the use of transmission impairment factors that reflect the effects of modern signal processing devices. This revision provides an enhanced version of the E-model to better take into account the effects of room noise at the send side, and quantizing distortion.

- Source ITU-T Recommendation G.107 was revised by ITU-T Study Group 12 (1997-2000) and approved under the WTSC Resolution 1 procedure on 18 May 2000.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSC Resolution 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	General		1

1.1	Scope		1

1.2	References		1

2	The E-model, a computational model for use in transmission planning		2

2.1	Introduction		2

2.2	Source code		2

3	The structure and basic algorithms of the E-model		2

3.1	Calculation of the transmission rating factor, R		3

3.2	Basic signal-to-noise ratio, Ro		4

3.3	Simultaneous impairment factor, Is		4

3.4	Delay impairment factor, Id		5

3.5	Equipment impairment factor, Ie		7

3.6	Advantage factor, A		7

3.7	Default values		7

Annex A  Conditions of using the E-model		8

Annex B  Quality measures derived from the transmission rating factor R		10

Annex C  Source code for G.107_2 in BASIC		12

Appendix I  Bibliography		16

ITU-T Recommendation G.107

The E-model, a computational model for use in transmission planning

# 1	General

## 1.1	Scope

This ITU-T Recommendation describes a computational model, known as the E-model, that has proven useful as a transmission planning tool, for assessing the combined effects of variations in several transmission parameters that affect conversational quality of 3.1 kHz handset telephony. This computational model can be used, for example, by transmission planners to help ensure that users will be satisfied with end-to-end transmission performance whilst avoiding over-engineering of networks. It must be emphasized that the primary output from the model is the "Rating Factor" R but this can be transformed to give estimates of customer opinion. Such estimates are only made for transmission planning purposes and not for actual customer opinion prediction (for which there is no agreed-upon model recommended by the ITU-T). Accordingly, the model described here is intended to be used to do relative comparisons of transmission conditions.

This revision provides an enhanced version of the E-model to better take into account the effects of room noise at the send side, and quantizing distortion. Because of this users should be aware that the computational algorithm has changed slightly  see Annex C.

The E-model has not been fully verified by field surveys or laboratory tests for the very large number of possible combinations of input parameters. For many combinations of high importance to transmission planners, the E-model can be used with confidence, but for other parameter combinations, E-model predictions have been questioned and are currently under study. Accordingly, caution must be exercised when using the E-model for some conditions; for example, the E-model may give inaccurate results for combinations of certain types of impairments. Annex A provides further information in this regard.

## 1.2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation G.100 (1993), Definitions used in Recommendations on general characteristics of international telephone connections and circuits.

[2]	ITU-T Recommendation G.108 (1999), Application of the E-model: A planning guide.

[3]	ITU-T Recommendation G.109 (1999), Definition of categories of speech transmission quality.

[4]	ITU-T Recommendation G.113 (1996), Transmission impairments.

[5]	Recommendation G.113  Appendix I (1999), Transmission impairments  Appendix I: Provisional planning values for the equipment impairment factor Ie.

# 2	The E-model, a computational model for use in transmission planning

## 2.1	Introduction

The complexity of modern networks requires that for transmission planning the many transmission parameters are not only considered individually but also that their combination effects are taken into account. This can be done by "expert, informed guessing," but a more systematic approach is desirable, such as by using a computational model. The output from the model described here is a scalar quality rating value, R, which varies directly with the overall conversational quality. ITU-T Recommendation G.113 [4] gives guidance about specific impairments, including combinations effects based upon a simplification of the model. However, the output can also give nominal estimates of user reactions, for instance in the form of percentages finding the modelled connection "Good or Better" or "Poor or Worse", as described in Annex B. Furthermore, detailed guidance on the proper application of the E-model  as described in this ITU-T Recommendation  is provided in ITU-T Recommendation G.108 [2]. In addition, the definition of categories of speech transmission quality can be found in ITU-T Recommendation G.109 [3].

## 2.2	Source code

Annex C contains the source code in BASIC of the E-model described in this ITU-T Recommendation. The purpose of this code is to ensure that users of the E-model are using consistent implementations of the formulae.

# 3	The structure and basic algorithms of the E-model

The E-model is based on the equipment impairment factor method, following previous transmission rating models. It was developed by an ETSI ad hoc group called "Voice Transmission Quality from Mouth to Ear".

The reference connection, as shown in Figure 1, is split into a send side and in a receive side. The model estimates the conversational quality from mouth to ear as perceived by the user at the receive side, both as listener and talker.

Figure 1/G.107 – Reference connection of the E-model

The transmission parameters used as an input to the computation model are shown in Figure 1. Values for room noise and for the D-factors are handled separately in the algorithm for send side and receive side and may be of different amount. The parameters SLR, RLR and circuit noise Nc are referred to a defined 0 dBr point. All other input parameters are either considered as values for the overall connection such as OLR (in any case the sum of SLR and RLR), number of qdu, equipment impairment factors Ie and advantage factor A, or referred only to the receive side, such as STMR, LSTR, WEPL (for calculation of Listener Echo) and TELR.

There are three different parameters associated with transmission time. The absolute delay Ta represents the total one-way delay between send side and receive side and is used to estimate the impairment due to too-long delay. The parameter mean one-way delay T represents the delay between the receive side (in talking state) and the point in a connection where a signal coupling occurs as a source of echo. The round-trip delay Tr only represents the delay in a 4-wire loop, where the "double reflected" signal will cause impairments due to Listener Echo.

## 3.1	Calculation of the transmission rating factor, R

According to the equipment impairment factor method, the fundamental principle of the E-model is based on a concept given in the description of the OPINE model [see Bibliography, Supplement 3 to P-series]:

Psychological Factors on the psychological scale are additive.

The result of any calculation with the E-model in a first step is a transmission rating factor R, which combines all transmission parameters relevant for the considered connection. This rating factor R is composed of:

R=Ro−Is−Id−Ie+A	(1)

Ro represents in principle the basic signal-to-noise ratio, including noise sources such as circuit noise and room noise. The factor Is is a combination of all impairments which occur more or less simultaneously with the voice signal. Factor Id represents the impairments caused by delay and the equipment impairment factor Ie represents impairments caused by low bit rate codecs. The advantage factor A allows for compensation of impairment factors when there are other advantages of access to the user. The term Ro and the Is and Id values are subdivided into further specific impairment values. The following subclauses give the formulae used in the E-model.

## 3.2	Basic signal-to-noise ratio, Ro

The basic signal-to-noise ratio Ro is defined by:

Ro=15−1.5SLR+No	(2)

The term No [in dBm0p] is the power addition of different noise sources:

No=10 lg 10Nc10+10Nos10+10Nor10+10Nfo10	(3)

Nc [in dBm0p] is the sum of all circuit noise powers, all referred to the 0 dBr point.

Nos [in dBm0p] is the equivalent circuit noise at the 0 dBr point, caused by the room noise Ps at the send side:

Nos=Ps−SLR−Ds−100+0.004Ps−OLR−Ds−142	(4)

where OLR  SLR  RLR. In the same way the room noise Pr at the receive side is transferred into an equivalent circuit noise Nor [in dBm0p] at the 0 dBr point.

Nor=RLR−121+Pre+0.008(Pre−35)2	(5)

The term Pre [in dBm0p] is the "effective room noise" caused by the enhancement of Pr by the listener's sidetone path:

Pre=Pr +10 lg 1+10(10–LSTR)10	(6)

Nfo [in dBm0p] represents the "noise floor" at the receive side,

Nfo=Nfor+RLR	(7)

with Nfor usually set to 64 dBmp.

## 3.3	Simultaneous impairment factor, Is

The factor Is is the sum of all impairments which may occur more or less simultaneously with the voice transmission. The factor Is is divided into three further specific impairment factors:

Is=Iolr+Ist+Iq	(8)

Iolr represents the decrease in quality caused by too-low values of OLR and is given by:

Iolr=20 1+X8818–X8	(9)

where:

X=OLR+0.2(64+No−RLR)	(10)

The factor Ist represents the impairment caused by non-optimum sidetone:

Ist=10 1+STMRo–125616–46 1+STMRo2310110+ 36	(11)

where:

STMRo=–10 lg 10−STMR10+ e−T4 10−TELR10	(12)

The impairment factor Iq represents impairment caused by quantizing distortion:

Iq=15 lg 1+10Y+10Z	(13)

where:

Y=Ro−10015+468.4−G9	(14)

Z=4630−G40	(15)

and:

G=1.07+0.258Q+0.0602Q2	(16)

Q=37−15 lg(qdu)	(17)

In this formula, qdu means the number of qdu for the whole connection between send side and receive side.

NOTE – If an impairment factor Ie is used for a piece of equipment, then the qdu value for that same piece of equipment must not be used.

## 3.4	Delay impairment factor, Id

Also Id, the impairment factor representing all impairments due to delay of voice signals is further subdivided into the three factors Idte, Idle and Idd:

Id=Idte+Idle+Idd	(18)

The factor Idte gives an estimate for the impairments due to Talker Echo:

Idte=Roe–Re2+(Roe–Re)24+100–11–e–T	(19)

where:

Roe=−1.5(No−RLR)	(20)

Re=80+2.5(TERV−14)	(21)

TERV=TELR–40 lg 1+T101+T150+6e–0.3T2	(22)

For values of T  1 ms, the Talker Echo should be considered as sidetone, i.e. Idte  0. The computation algorithm furthermore combines the influence of STMR to Talker Echo. Taking into account that low values of STMR may have some masking effects on the Talker Echo and for very high values of STMR the Talker Echo may become more noticeable, the terms TERV and Idte are adjusted as follows:

For STMR  9 dB:

In equation (20) TERV is replaced by TERVs, where:

TERVs=TERV+Ist2	(23)

For 9 dB  STMR  15 dB:

the above-given equations (18) to (21) apply.

For STMR  15 dB:

Idte is replaced by Idtes, where:

Idtes=Idte2+Ist2	(24)

The factor Idle represents impairments due to Listener Echo. The equations are:

Idle=Ro–Rle2+(Ro–Rle)24+169	(25)

where:

Rle=10.5(WEPL+7)(Tr+1)–0.25	(26)

The factor Idd represents the impairment caused by too-long absolute delay Ta, which occurs even with perfect echo cancelling.

For Ta  100 ms:

Idd=0

For Ta  100 ms:

Idd=251+X616–31+X3616+ 2	(27)

with:

X=lgTa100lg 2	(28)

## 3.5	Equipment impairment factor, Ie

The values for the Equipment Impairment Factor Ie of elements using low bit rate codecs are not related to other input parameters. They are depending on subjective mean opinion score test results as well as on network experience. Refer to Appendix I/G.113 [5] for the actually recommended values of Ie.

## 3.6	Advantage factor, A

Due to the specific meaning of the advantage factor A, there is – consequently – no relation to all other transmission parameters. Some provisional values are given in Table 1.

Table 1/G.107 – Provisional examples for the advantage factor A

| Communication system example | Maximum value of A |
| --- | --- |
| Conventional (wirebound) | 0 |
| Mobility by cellular networks in a building | 5 |
| Mobility in a geographical area or moving in a vehicle | 10 |
| Access to hard-to-reach locations, e.g. via multi-hop satellite connections | 20 |

It should be noted that the values in Table 1, taken from ITU-T Recommendation G.113 [4], are only provisional. The use of the factor A and its selected value in a specific application is up to the planner's decision. However, the values in Table 1 should be considered as absolute upper limits for A.

## 3.7	Default values

For all input parameters used in the algorithm of the E-model, the default values are listed in Table 2. It is strongly recommended to use these default values for all parameters which are not varied during planning calculation. If all parameters are set to the default values, the calculation results in a very high quality with a rating factor of R  93.2.

Table 2/G.107 – Default values and permitted ranges for the parameters

| Parameter | Abbr. | Unit | Default value | Permitted range | Remark |
| --- | --- | --- | --- | --- | --- |
| Send Loudness Rating | SLR | dB | +8 | 0 ... +18 | Note 1 |
| Receive Loudness Rating | RLR | dB | +2 | 5 ... +14 | Note 1 |
| Sidetone Masking Rating | STMR | dB | 15 | 10 ... 20 | Note 2 |
| Listener Sidetone Rating | LSTR | dB | 18 | 13 ... 23 | Note 2 |
| D-Value of Telephone, Send Side | Ds | – | 3 | –3 ... +3 | Note 2 |
| D-Value of Telephone Receive Side | Dr | – | 3 | 3 ... +3 | Note 2 |
| Talker Echo Loudness Rating | TELR | dB | 65 | 5 ... 65 |  |
| Weighted Echo Path Loss | WEPL | dB | 110 | 5 ... 110 |  |
| Mean one-way Delay of the Echo Path | T | msec | 0 | 0 ... 500 |  |
| Round Trip Delay in a 4-wire Loop | Tr | msec | 0 | 0 ... 1000 |  |
| Absolute Delay in echo-free Connections | Ta | msec | 0 | 0 ... 500 |  |

Table 2/G.107 – Default values and permitted ranges for the parameters (concluded)

| Parameter | Abbr. | Unit | Default value | Permitted range | Remark |
| --- | --- | --- | --- | --- | --- |
| Number of Quantization Distortion Units | qdu | – | 1 | 1 ... 14 |  |
| Equipment Impairment Factor | Ie | – | 0 | 0 ... 40 |  |
| Circuit Noise referred to 0 dBr-point | Nc | dBm0p | 70 | 80 ... 40 |  |
| Noise Floor at the Receive Side | Nfor | dBmp | 64 | – | Note 3 |
| Room Noise at the Send Side | Ps | dB(A) | 35 | 35 ... 85 |  |
| Room Noise at the Receive Side | Pr | dB(A) | 35 | 35 ... 85 |  |
| Advantage Factor | A | – | 0 | 0 ... 20 |  |
| NOTE 1 – Total values between microphone or receiver and 0 dBr-point. NOTE 2 – Fixed relation: LSTR = STMR + D. NOTE 3 – Currently under study. |  |  |  |  |  |

The year 2000 revision of this ITU-T Recommendation provides an enhanced version of the E-model algorithm (see Annex A).

Due to this revision the resulting rating R with all parameter values default has slightly changed (from R = 94.2 to R = 93.2). For practical planning purposes, however, this slight deviation should be considered insignificant.

ANNEX A

Conditions of using the E-model

NOTE  The assessment and enhancement of the E-model algorithm will be for further study during the Study Period 2001-2004. New results will be included as soon as they become available.

Part I: Examples of conditions where caution must be exercised when using the E-model

	The overall level of the equipment impairment factors

Some experimental investigations suggest that the general tendency of the equipment impairment factors is too pessimistic, so that a hidden security margin may be incorporated.

	The overall additivity property of the model

The E-model supposes that different kinds of impairments are additive on the scale of the transmission rating factor R. This feature has not been checked to a satisfying extent. Especially, very few investigations are available regarding the interaction of low bit rate codecs with other kinds of impairments, e.g. with room noise. Additionally, the order effects when tandeming several low bit rate codecs remain uncertain.

	The coverage of talker sidetone

Some experiments show that the E-model disregards some masking effects occurring for talker sidetone, namely in conjunction with circuit noise, room noise at receive side, and low delay talker echo (<10 ms).

	The advantage factor A

Up to now it has not been clarified under which conditions the given values for the advantage factor should be applied. It is expected that these values may depend, e.g. on the user group, and that the absolute values will change in long term.

	Derivation methodology for new equipment impairment factors

A new methodology for deriving equipment impairment factors is currently under study within ITU-T SG 12.

	Predictions for different types of room noise, and different frequency shapes in the communication channel, in the sidetone path and in the echo path

The E-model regards the effect of room noise only by means of an A-weighted level. The actual opinion on the speech communication quality may depend even on the type and disturbance of the environmental noise. The frequency characteristics of the communication channel, of the sidetone and of the echo path are not explicitly regarded by the E-model, but only implicitly by means of loudness ratings. However, they may affect the perceived transmission quality.

Part II: Conditions for which the performance of the E-model has been improved by updating from the earlier version

	The effect of room noise at send side

With the present enhanced E-model algorithm (year 2000 revision) the Lombard effect (the fact that the speaker adopts his/her pronunciation and speaking level to the noise environment) is no longer disregarded. This has  in the 1998 version  lead to too pessimistic E-model predictions for high room noise levels Pr.

	Predictions for quantizing distortion

In case of the 1998 version of the E-model, subjective test results for MNRU reference conditions were very often more pessimistic than E-model predictions. The graphs in Figure A.1 have been derived from the 1998 version and the year 2000 revision of the E-model with all other parameters at their default values.

Figure A.1/G.107  Relation between the number of qdu and E-model Rating R

With respect to the slightly enhanced algorithm of the E-model as given in the present Recommendation, the relation between the Parameter qdu and the E-model Rating R has been changed in order to align the algorithm better with the available subjective test results.

ANNEX B

Quality measures derived from the transmission rating factor R

The transmission rating factor R can lie in the range from 0 to 100, where R  0 represents an extremely bad quality and R  100 represents a very high quality. The E-model provides a statistical estimation of quality measures. The percentages for a judgement Good or Better (GoB) or Poor or Worse (PoW) are obtained from the R-factor by means of the Gaussian Error function:

E(x)=12π–∞xe–t22dt	(B-1)

The equations are:

GoB=100ER–6016 %	(B-2)

PoW=100E45–R16 %	(B-3)

The Mean Opinion Score (MOS) in the scale 1-5 can be obtained from the R-factor using the formulae:

For R  0:	MOS=1

For 0  R  100:	MOS=1+0.035R+R(R−60)(100−R)7⋅10−6	(B-4)

For R  100:	MOS=4.5

GoB, PoW and MOS as functions of R are depicted in Figures B.1 and B.2 respectively.

Figure B.1/G.107 – GoB (Good or Better) and PoW (Poor or Worse) 
as functions of rating factor R

Figure B.2/G.107 – MOS as function of rating factor R

In some cases, transmission planners may not be familiar with the use of quality measures such as the R rating factor obtained from planning calculations, and thus provisional guidance for interpreting calculated R factors for planning purposes is given in Table B.1. This table also contains equivalent transformed values of R into MoS, GOB and PoW.

Table B.1/G.107 – Provisional guide for the relation between R-value and user satisfaction

| R-value (lower limit) | MOS (lower limit) | GoB (%) (lower limit) | PoW (%) (upper limit) | User satisfaction |
| --- | --- | --- | --- | --- |
| 90 | 4.34 | 97 | 0 | Very satisfied |
| 80 | 4.03 | 89 | 0 | Satisfied |
| 70 | 3.60 | 73 | 6 | Some users dissatisfied |
| 60 | 3.10 | 50 | 17 | Many users dissatisfied |
| 50 | 2.58 | 27 | 38 | Nearly all users dissatisfied |

ANNEX C

Source code for G.107_2 in BASIC

1  CLS

2  PRINT "PROGRAM g107_2"

3  REM THIS VERSION IS CONFORM WITH THE ALGORITHM

4  REM DESCRIBED IN REC. G.107

5  REM PROGRAM WRITTEN BY N.O. JOHANNESSON

6  REM LAST MODIFICATIONS BY S. MOELLER, July 1999

7  PRINT

8  PRINT "E-model, algorithm according to ITU-T Rec. G.107 (05/00) Annex C,"

9  PRINT "for voice communication between side (S) and (R)."

10 PRINT

11 PRINT "The E-model gives an estimation of voice transmission quality"

12 PRINT "mouth-to-ear as perceived at Side (R). The model output is a"

13 PRINT "transmission rating factor R which can be transformed to give"

14 PRINT "estimates of customer opinion. Such estimates are only made for"

15 PRINT "transmission planning purposes and not for actual customer"

16 PRINT "opinion prediction. See ITU-T Rec. G.107 (05/00) for more details."

17 PRINT

18 PRINT "Note: To continue program:"

19 PRINT "a) INPUT appropriate value as indicated on screen,"

20 PRINT "or"

21 PRINT "b) if no value is asked for, PRESS THE RETURN KEY."

22 PRINT

23 DIM X(100), Y(5, 100), Col(5), Pxo(5)

24 REM Set parameters at default values

25 GOSUB 29

26 REM Goto main menu

27 GOTO 199

29 REM Default Parameter Values, according to Tab.3/G.107 (lines 29-50)

30 SLR = 8

31 RLR = 2

32 STMRs = 15

33 Ds = 3

34 STMR = 15

35 Dr = 3

36 LSTRr = 18

37 TELR = 65

38 T = 0

39 WEPL = 110

40 Tr = 0

41 Ta = 0

42 Ie = 0

43 A = 0

44 Nc = -70

45 Ps = 35

46 Pr = 35

47 qdu = 1

48 Nfor = -64

50 RETURN

198 REM Main menu (lines 198-370)

199 PRINT

200 PRINT "Print current parameter values ", "=1"

210 PRINT "Input new parameter values     ", "=2"

220 PRINT "Compute table, one parameter   ", "=3"

230 PRINT "Set parameter at default values", "=4"

240 PRINT "Exit program                   ", "=5"

250 PRINT

260 INPUT Y1

270 CLS

280 IF Y1 = 1 THEN GOSUB 500

290 IF Y1 = 2 THEN GOSUB 1000

300 IF Y1 = 3 THEN GOSUB 2000

310 IF Y1 = 4 THEN GOSUB 30

320 IF Y1 = 5 THEN GOTO 9999

330 CLS

340 IF Y1 = 4 THEN PRINT , "Parameters set at default values !"

350 GOTO 199

500 REM SUB Print current parameter values (lines 500-700)

510 PRINT , "SLR="; SLR, "RLR="; RLR, "OLR= SLR + RLR="; SLR + RLR

520 PRINT , "Side (S): Ds="; Ds

530 PRINT , "Side (R): STMR="; STMR, "Dr="; Dr, "LSTR="; STMR + Dr

540 PRINT

550 PRINT , "TELR="; TELR, "Mean One-way Delay T ms="; T

560 PRINT , "WEPL="; WEPL, "Round-trip Delay Tr ms="; Tr

570 PRINT , "One-way Absolute Delay Ta ms="; Ta

580 PRINT

590 PRINT , "Noise Floor at Side (R) Nfor dBmp="; Nfor

600 PRINT , "Circuit Noise Nc dBm0p="; Nc

610 PRINT , "Room Noise, Side (S), Ps dB(A)="; Ps

620 PRINT , "Room Noise, Side (R), Pr dB(A)="; Pr

630 PRINT

640 PRINT , "qdu="; qdu

650 PRINT

660 PRINT , "Equipment Impairment Factor Ie="; Ie

670 PRINT , "Advantage Factor A="; A

680 PRINT

690 INPUT C$

700 RETURN

1000 REM SUB Input Parameters (lines 1000-1270)

1020 CLS

1030 PRINT "Type designation of parameter for which the value is to be changed !"

1031 PRINT

1032 PRINT "Note 1. New value of OLR is obtained indirectly, i.e. by new"

1033 PRINT "value of SLR or RLR. (OLR=SLR+RLR.)"

1034 PRINT

1035 PRINT "Note 2. New value of LSTR is obtained indirectly, i.e. by new"

1036 PRINT "value of STMR or Dr. (LSTR=STMR+Dr.)"

1037 PRINT

1040 INPUT "Parameter:"; A$

1050 INPUT "New Value="; Px

1060 PRINT A$; "="; Px

1070 IF ((A$ = "SLR") OR (A$ = "slr") OR (A$ = "Slr")) THEN SLR = Px

1080 IF ((A$ = "RLR") OR (A$ = "rlr") OR (A$ = "Rlr")) THEN RLR = Px

1090 IF ((A$ = "STMR") OR (A$ = "stmr") OR (A$ = "Stmr")) THEN STMR = Px

1100 IF ((A$ = "Dr") OR (A$ = "DR") OR (A$ = "dr")) THEN Dr = Px

1110 IF ((A$ = "Ds") OR (A$ = "DS") OR (A$ = "ds")) THEN Ds = Px

1120 IF ((A$ = "TELR") OR (A$ = "telr") OR (A$ = "Telr")) THEN TELR = Px

1130 IF ((A$ = "T") OR (A$ = "t")) THEN T = Px

1140 IF ((A$ = "WEPL") OR (A$ = "wepl") OR (A$ = "Wepl")) THEN WEPL = Px

1150 IF ((A$ = "Tr") OR (A$ = "TR") OR (A$ = "tr")) THEN Tr = Px

1160 IF ((A$ = "Ta") OR (A$ = "TA") OR (A$ = "ta")) THEN Ta = Px

1170 IF ((A$ = "Ie") OR (A$ = "IE") OR (A$ = "ie")) THEN Ie = Px

1180 IF ((A$ = "A") OR (A$ = "a")) THEN A = Px

1190 IF ((A$ = "Nc") OR (A$ = "NC") OR (A$ = "nc")) THEN Nc = Px

1200 IF ((A$ = "Ps") OR (A$ = "PS") OR (A$ = "ps")) THEN Ps = Px

1210 IF ((A$ = "Pr") OR (A$ = "PR") OR (A$ = "pr")) THEN Pr = Px

1220 IF ((A$ = "qdu") OR (A$ = "QDU") OR (A$ = "Qdu")) THEN qdu = Px

1230 IF ((A$ = "Nfor") OR (A$ = "NFOR") OR (A$ = "nfor")) THEN Nfor = Px

1240 PRINT

1250 IF Y1 = 2 THEN INPUT "More parameters changed, Yes(1) or No(0)"; Ypar

1260 IF Ypar = 1 THEN GOTO 1020

1270 RETURN

2000 REM SUB Tabulate (lines 2000-3000)

2020 INPUT "Variable Parameter:"; A$

2030 PRINT "(To exit tabulation, put parameter value = 1000 !)"

2040 PRINT TAB(8); A$; TAB(18); "R"; TAB(28); "GOB %"; TAB(38); "POW %"; TAB(48); "MOS"

2050 INPUT Px

2060 IF Px = 1000 THEN GOTO 3000

2070 IF ((A$ = "SLR") OR (A$ = "slr") OR (A$ = "Slr")) THEN SLR = Px

2080 IF ((A$ = "RLR") OR (A$ = "rlr") OR (A$ = "Rlr")) THEN RLR = Px

2090 IF ((A$ = "STMR") OR (A$ = "stmr") OR (A$ = "Stmr")) THEN

2100   STMR = Px

2110   LSTR = STMR + Dr

2120 END IF

2130 IF ((A$ = "Dr") OR (A$ = "DR") OR (A$ = "dr")) THEN

2140   Dr = Px

2150   LSTR = STMR + Dr

2160 END IF

2170 IF ((A$ = "TELR") OR (A$ = "telr") OR (A$ = "Telr")) THEN TELR = Px

2180 IF ((A$ = "T") OR (A$ = "t")) THEN T = Px

2190 IF ((A$ = "WEPL") OR (A$ = "wepl") OR (A$ = "Wepl")) THEN WEPL = Px

2200 IF ((A$ = "Tr") OR (A$ = "TR") OR (A$ = "tr")) THEN Tr = Px

2210 IF ((A$ = "Ta") OR (A$ = "TA") OR (A$ = "ta")) THEN Ta = Px

2220 IF ((A$ = "Ie") OR (A$ = "IE") OR (A$ = "ie")) THEN Ie = Px

2230 IF ((A$ = "A") OR (A$ = "a")) THEN A = Px

2240 IF ((A$ = "Nc") OR (A$ = "NC") OR (A$ = "nc")) THEN Nc = Px

2245 IF ((A$ = "Nfor") OR (A$ = "NFOR") OR (A$ = "nfor")) THEN Nfor = Px

2250 IF ((A$ = "Ps") OR (A$ = "PS") OR (A$ = "ps")) THEN Ps = Px

2260 IF ((A$ = "Pr") OR (A$ = "PR") OR (A$ = "pr")) THEN Pr = Px

2270 IF ((A$ = "qdu") OR (A$ = "QDU") OR (A$ = "Qdu")) THEN qdu = Px

2280 IF ((A$ = "Ie") OR (A$ = "IE") OR (A$ = "ie")) THEN Ie = Px

2290 IF ((A$ = "Ds") OR (A$ = "DS") OR (A$ = "ds")) THEN Ds = Px

2300 GOSUB 3500

2400 GOSUB 4000

2500 GOSUB 4100

2600 GOSUB 4200

2700 R = INT(R * 10 + .5) / 10

2800 PRINT TAB(8); Px; TAB(18); R; TAB(28); GOB; TAB(38); POW; TAB(48); MOS

2900 GOTO 2050

3000 RETURN

3500 REM Compute R (lines 3500-3880)

3509 REM Noise Summation, formulas (3) to (7)

3510 Nr1 = Ps - SLR - Ds - 100

3520 Nr1 = Nr1 + .004 * (Ps - SLR - RLR - Ds - 14) ^ 2

3530 LSTR = STMR + Dr

3540 Pro = Pr + 10 * LOG(1 + 10 ^ ((10 - LSTR) / 10)) / LOG(10)

3550 Pr1 = Pro + .008 * (Pro - 35) ^ 2

3560 Nr2 = Pr1 - 121 + RLR

3570 Nfo = Nfor + RLR

3580 No = 10 * LOG(10 ^ (Nr1 / 10) + 10 ^ (Nr2 / 10) + 10 ^ (Nc / 10) + 10 ^ (Nfo / 10)) / LOG(10)

3590 Nt = No - RLR

3599 REM Ro, formula (2)

3600 Ro = 15 - 1.5 * (SLR + No)

3609 REM Iolr, formulas (9) and (10)

3610 Xolr = SLR + RLR + .2 * (64 + Nt)

3620 Iolr = 20 * ((1 + (Xolr / 8) ^ 8) ^ (1 / 8) - Xolr / 8)

3629 REM Ist, formulas (11) and (12)

3630 STMRo = -10 * LOG(10 ^ (-STMR / 10) + 10 ^ (-TELR / 10) * EXP(-T / 4)) / LOG(10)

3640 Ist = 10 * (1 + ((STMRo - 12) / 5) ^ 6) ^ (1 / 6) - 10

3650 Ist = Ist - 46 * (1 + (STMRo / 23) ^ 10) ^ (1 / 10) + 46

3659 REM Iq, formulas (13) to (17)

3660 IF qdu < 1 THEN qdu = 1

3670 Q = 37 - 15 * LOG(qdu) / LOG(10)

3680 G = 1.07 + .258 * Q + .0602 * Q ^ 2

3690 Iq = 15 * LOG(1 + 10 ^ ((Ro - 100) / 15) * 10 ^ (46 / 8.4 - G / 9) + 10 ^ (46 / 30 - G / 40)) / LOG(10)

3699 REM Is, formula (8)

3700 Isyn = Iolr + Ist + Iq

3709 REM TERV, formula (22)

3710 TERV = TELR + 6 * EXP(-.3 * T ^ 2) - 40 * LOG((1 + T / 10) / (1 + T / 150)) / LOG(10)

3719 REM Modifications to satisfy formula (23)

3720 IF STMR < 9 THEN TERV = TERV + .5 * Ist

3729 REM Idte, formulas (19) to (21)

3730 Re = 80 + 2.5 * (TERV - 14)

3740 Roe = -1.5 * (No - RLR)

3750 Xdt = (Roe - Re) / 2

3760 Idte = Xdt + SQR(Xdt ^ 2 + 100)

3770 Idte = (Idte - 1) * (1 - EXP(-T))

3779 REM Modifications to satisfy formula (24)

3780 IF STMR > 15 THEN Idte = SQR(Idte ^ 2 + Ist ^ 2)

3789 REM Idle, formulas (25) and (26)

3790 Rle = 10.5 * (WEPL + 7) * (Tr + 1) ^ (-1 / 4)

3800 Xdl = (Ro - Rle) / 2

3810 Idle = Xdl + SQR(Xdl ^ 2 + 169)

3819 REM Idd, formulas (27) and (28)

3820 IF Ta < 100 THEN Idd = 0

3830 IF Ta = 100 THEN Idd = 0

3840 IF Ta > 100 THEN

X = (LOG(Ta / 100)) / LOG(2)

Idd = 25 * ((1 + X ^ 6) ^ (1 / 6) - 3 * (1 + (X / 3) ^ 6) ^ (1 / 6) + 2)

3850 END IF

3859 REM Id

3860 Id = Idte + Idle + Idd

3869 REM R, formula (1)

3870 R = Ro - Isyn - Id - Ie + A

3880 RETURN

4000 REM Compute GOB, formula (B.2) (lines 4000-4050)

4010 Z# = (R - 60) / 16

4020 GOSUB 5000

4030 GOB = 100 * F#

4040 GOB = INT(GOB * 10 + .5) / 10

4050 RETURN

4100 REM Compute POW, formula (B.3) (lines 4100-4150)

4110 Z# = (R - 45) / 16

4120 GOSUB 5000

4130 POW = 100 * (1 - F#)

4140 POW = INT(POW * 10 + .5) / 10

4150 RETURN

4200 REM Compute MOS, formula (B.4) (lines 4200-4260)

4210 MOS = 1 + R * .035 + R * (R - 60) * (100 - R) * 7 * 10 ^ (-6)

4220 MOS = INT(MOS * 100 + .5) / 100

4230 IF R < 0 THEN MOS = 1

4240 IF MOS < 1 THEN MOS = 1

4250 IF R > 100 THEN MOS = 4.5

4260 RETURN

5000 REM Norm Distr F(Z), formula (B.1) (lines 5000-5130)

5010 S# = 0

5020 N% = 0

5030 H# = Z#

5040 S# = S# + H#

5050 H# = H# * (-1) * (Z#) ^ 2 * (2 * N% + 1) / ((N% + 1) * 2 * (2 * N% + 3))

5060 N% = N% + 1

5070 IF ABS(H#) < 10 ^ (-6) THEN GOTO 5090

5080 GOTO 5040

5090 S# = S# / (SQR(2 * 3.14159265#))

5100 F# = .5 + S#

5110 F# = INT(F# * 10 ^ 5 + .5) / 10 ^ 5

5120 REM PRINT "Z="; Z#, "F(Z)="; F#, "N="; N%

5130 RETURN

9999 END

Appendix I

Bibliography

–	ITU-T Recommendation G.107 (1998), The E-model, a computational model for use in transmission planning.

–	Supplement 3 to ITU-T Series P Recommendations (1988), Models for predicting transmission quality from objective measurements.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |

Conversational quality in this context refers to transmission characteristics, e.g. long transmission times, effects of talker echoes, etc. However, the E-model as described in this ITU-T Recommendation is not intended to model transmission impairments during double talk situations.

The source of Table B.1 is Table 1/G.109 [3].